Open Mission Systems (OMS)

Definition And Documentation (D&D)

Language-Agnostic (LA) Critical Abstraction Layer (CAL) Specification

22 January 2026

Prepared By:

Open Architecture Collaborative Working Group (OACWG)

This page is intentionally left blank.

Abstract

Open Mission Systems (OMS) is a non-proprietary, open architecture standard for integrating subsystems and services into mission packages.

This document defines the required attributes and behaviors for implementations of the Open Mission Systems (OMS) Language-Agnostic (LA) Critical Abstraction Layer (CAL) Application Programming Interface (API). A provider of an OMS Platform is responsible for delivering required CAL Server implementations with well-defined CAL APIs. The required attributes and behaviors defined by this document provide commonality between disparate CAL Server implementations, regardless of the messaging technology stack selected by the implementer. This document makes no restriction on the tools, libraries, or products that may be used to meet the requirements and fully implement the CAL API.

This page is intentionally left blank.

Revision Record

| REVISION | DATE | DESCRIPTION |
| --- | --- | --- |
| B | 22 January 2026 | Initial public release |

This page is intentionally left blank.

Table of Contents

1	Introduction	1

2	Scope	2

3	References	3

3.1	OMS D&D Documents	3

3.2	Other Documents	3

4	CAL Terms and Definitions	4

4.1	OMS Messages	4

4.2	CAL Messages	5

4.3	CAL Client	5

4.4	CAL Server	5

4.5	Network Configuration	5

4.6	Service Identifier	6

4.7	Topics and Connections	6

4.8	Subscription Groups	7

5	Protocols	8

5.1	The WebSocket Protocol	8

5.1.1	Subprotocols	9

5.1.1.1	OMS WebSocket Protocol	9

5.1.1.1.1	Core Concepts	11

5.1.1.1.2	Protocol Conventions	12

5.1.1.1.3	Protocol Operations	13

5.1.1.1.4	OWP External References	20

6	CAL Message Formats	21

6.1	OMS JSON	21

6.1.1	Global Element Declarations	21

6.1.1.1	Examples	21

6.1.2	Particles	23

6.1.3	Complex Type Definitions	24

6.1.3.1	Examples	24

6.1.4	Simple Type Definitions	30

6.1.4.1	Examples	31

6.1.5	Validation	32

6.1.5.1	Document Information Item	33

6.1.5.2	Namespace Name and Local Name of a Member	33

6.1.5.3	Element Information Items of a Member	34

6.1.5.4	Character Information Items of Values	34

6.1.6	OMS JSON External References	35

7	Acronyms and Symbols	37

8	Glossary	38

This page is intentionally left blank.

List of Figures

Figure 4.0-1 CAL Client Stack	4

Figure 4.7-1 CAL and Client Topics	6

Figure 4.8-1 Subscription Groups	7

This page is intentionally left blank.

List of Tables

Table 3.1-1 OMS D&D Documents	3

Table 3.2-1 Other Documents	3

Table 5.0-1 Protocols	8

Table 5.1-1 Subprotocols	9

Table 5.1-2 OWP CAL Message Formats	10

Table 5.1-3 Protocol Operations	13

Table 5.1-4 INIT Object	14

Table 5.1-5 INFO Object	17

Table 5.1-6 Identifiers Object	18

Table 5.1-7 Errors	19

Table 5.1-8 OWP External References	20

Table 6.1-1 Document Information Item	33

Table 6.1-2 Element Information Items of a Member	34

Table 6.1-3 Character Information Items of Values	35

Table 6.1-4 OMS JSON External References	35

This page is intentionally left blank.

# Introduction

The Language-Agnostic (LA) Critical Abstraction Layer (CAL) Specification enables the use of modern programming languages and provides a non-proprietary interface for cloud-based systems to communicate with other CAL Clients on a Platform. This specification exists as an alternative to the language-dependent CAL APIs of C++ and Java. The LA-CAL Specification defines standard interactions and behaviors for OMS Messaging by CAL Clients and CAL Servers through a CAL API, replacing the need for ad-hoc adapters. Protocols standardize communication between CAL Servers and CAL Clients. Unlike with language-dependent CAL APIs, CAL Clients that use an LA-CAL are responsible for UUID generation and message construction. LA-CAL uses language-independent data interchange formats to represent its CAL Messages as opposed to the CAL Message objects constructed by language-dependent CAL APIs. Each data interchange format follows its own grammar resulting in well-formed CAL Messages.

# Scope

In the following sections, the LA-CAL Specification requirements are described and enumerated as certification requirements (CERTs). Guidance and descriptive text have been added to assist the developer in achieving the goals of this specification and to help verification engineers fully understand the intention of the requirements and how they should be tested.

Terms and definitions in this document should be considered first when other OMS specifications provide overlapping terms or definitions, except for those defined in OMSC-STD-001, OMS Standard, and OMSC-STD-002, Abbreviations and Glossary.

# References

## OMS D&D Documents

Table 3.1-1 OMS D&D Documents

| Document Number | Document Title | Revision | Date |
| --- | --- | --- | --- |
| OMSC-STD-001 | OMS Standard Version 2.5 | M | 22 January 2026 |
| OMSC-STD-002 | Abbreviations and Glossary | M | 22 January 2026 |
| OMSC-TMP-001 | Platform Description Document (PDD) Template | M | 22 January 2026 |
| OMSC-TMP-003 | Service Contract Template | M | 22 January 2026 |

## Other Documents

Table 3.2-1 Other Documents

| Document Number | Document Title | Revision | Date |
| --- | --- | --- | --- |
| OAC-SPC-001 | Universal Command and Control (C2) Interface (UCI) Schema Style and Design Specification | E | 22 January 2026 |
| IETF RFC 4122 | A Universally Unique IDentifier (UUID) URN Namespace |  | July 2005 |
| IETF RFC 6455 | The WebSocket Protocol |  | December 2011 |
| IETF RFC 8259 | The JavaScript Object Notation (JSON) Data Interchange Format |  | December 2017 |
| N/A | XML Schema Part 1: Structures Second Edition | 1.0 | 28 October 2004 |
|  | Namespaces in XML 1.0 (Third Edition) |  | 08 December 2009 |
|  | XML Information Set (Second Edition) |  | 04 February 2004 |

# CAL Terms and Definitions

The Critical Abstraction Layer (CAL) is the OMS-defined Application Programming Interface (API) through which CAL Clients interact with a CAL Server as shown in Figure 4.0-1, CAL Client Stack. This API includes interfaces for publishing and receiving CAL Messages. OMS messaging interactions between OMS participants must use the CAL-provided publish and subscribe interface.

Figure 4.0-1 CAL Client Stack

## OMS Messages

OMS Messages are defined in the OMS Message Schema, an eXtensible Markup Language (XML) Schema Definition (XSD) file. The OMS Message Schema is based on the Universal Command and Control (C2) Interface (UCI) message schema. The version of the UCI message schema and the definition of any OMS-specific schema extension are found in the OMSC-STD-001, OMS Standard.

## CAL Messages

CAL Messages are representations of OMS Messages (see Section 4.1, OMS Messages) in code according to a message format defined in Section 6, CAL Message Formats. A CAL Server (see Section 4.4, CAL Server) converts between CAL Messages and OMS Messages; a process sometimes called serialization/de-serialization. When a message is being manipulated by a CAL Client it is a CAL Message. When a message is in its serialized form it is an OMS Message. This distinction between CAL Messages and OMS Messages is used to prevent ambiguity in CERTs and descriptive text and to ensure that the CERTs are only levied against entities under the purview of the CAL. In other words, CERTs are not levied against OMS Messages (as defined in Section 4.1, OMS Messages) in this document.

## CAL Client

A CAL Client (see Figure 4.0-1, CAL Client Stack) uses a CAL Server to send and receive CAL Messages. Each CAL Client is uniquely identified by their Service Identifier (see Section 4.6, Service Identifier). Throughout this document the term "CAL Client" is used to indicate the software that interacts with a CAL Server. One type of CAL Client is an OMS Service; however, CAL Servers may be used by applications that are not fully compliant OMS Services.

## CAL Server

A CAL Server is a manifestation and fulfillment of the requirements defined in this specification. One or more CAL Servers serve as an integral component of an Abstract Service Bus (ASB); an ASB is a required element of an OMS Platform. A Platform may make available an appropriate CAL Server that satisfies this specification. A CAL Client (see Section 4.3, CAL Client) connects to a CAL Server using a protocol specified in Section 5, Protocols. CAL Servers may be implemented in any programming language and do not need to be in the same language as any CAL Client. For example, a CAL Client implemented in Python can connect to a CAL Server implemented in Rust. Even though any programming language may be used, developers must still be sensitive to the fact that there are unsigned data types in the schema and handle such types appropriately.

## Network Configuration

A CAL Server is configured at runtime to connect with underlying middleware technologies, network resources, and transport layers. The Network Configuration identifies valid CAL Topics, their associated network connections, and how these network connections are actually implemented across the available network resources.

## Service Identifier

A Service Identifier is a string name given to a Service that is used to associate it with a globally unique identifier called a UUID (Internet Engineering Task Force (IETF) Request for Comments (RFC) 4122) specific to an instance of that Service on a single specific system. By system, it is meant an element that could be represented as, e.g., a vehicle, a node acting as a collection of vehicles, or a node on a ground control system. The Service Identifier is a required parameter used when a CAL Client initializes a Connection to a CAL Server.

## Topics and Connections

A Connection is a communication link established between CAL Clients through an ASB. A CAL Topic is a CAL Server's internal identifier for a communications link between publications and subscriptions. Multiple Connections may be associated with a single CAL Topic: CAL Clients may have Connections to multiple subscriptions over a given CAL Topic; and subscriptions can have Connections to multiple CAL Clients over a given CAL Topic. A Client Topic is the logical topic name used by a CAL Client to identify the communications between publications and subscriptions. A CAL Server maps internal CAL Topics to Client Topics. Figure 4.7-1, CAL and Client Topics, depicts these interactions.

Figure 4.7-1 CAL and Client Topics

## Subscription Groups

A Subscription Group is a grouping of one or more subscriptions formed on a CAL Topic. Subscription Groups provide a mechanism for application fault tolerance and scalable workload processing. A Client Subscription Group is identified by name given by a CAL Client. A CAL Subscription Group is a CAL Server's internal mapping of a Client Subscription Group. Two subscriptions are members of the same CAL Subscription Group when the same CAL Subscription Group and CAL Topic are specified. CAL Subscription Groups with the same identifier are distinct if they are registered on different CAL Topics.

When a published message matches the topic of a Subscription Group, only one member of that group receives the message. This contrasts from typical publish and subscribe behavior in which all subscribers receive the message. Figure 4.8-1, Subscription Groups, shows CAL Clients B, C, and D having subscriptions forming a Subscription Group. Messages published from CAL Client A are distributed among the group.

Figure 4.8-1 Subscription Groups

# Protocols

A Language-Agnostic CAL is constructed as a layered protocol from other open standards. The following sections describe required behaviors and interactions required by an LA-CAL implementation to enable machine-to-machine communication aligning with OMS Standard.

CERT LACAL-000001 [An LA-CAL shall implement at least one protocol in the "Protocols" table.]

Table 5.0-1 Protocols

Protocol

The WebSocket Protocol

## The WebSocket Protocol

The WebSocket Protocol, defined in RFC 6455, enables two-way communication between a client and a remote server. The protocol consists of an opening handshake followed by basic message framing, layered over TCP. The protocol has two parts: a handshake and the data transfer. The handshake consists of an HTTP connection upgrade request, followed by an HTTP response from the server. After a successful handshake, clients and servers transfer data back and forth in conceptual units referred to in this specification as "WebSocket messages". On the wire, a WebSocket message is composed of one or more frames. Each frame belonging to the same WebSocket message contains the same type of data. All data is communicated as textual data, encoded as UTF-8 text.

RFC 6455 levies certain requirements on server implementations. These requirements are defined in RFC 6455 Section 4.2, Server-Side Requirements, and must be complied with by all CAL Server implementations.

CERT LACAL-000002 [An LA-CAL that supports the WebSocket Protocol shall implement the Server-Side Requirements as defined in RFC 6455.]

A CAL Server must require that the Sec-WebSocket-Protocol header field is present in the client's opening handshake, whereas in RFC 6455 Section 4.2.1, Reading the Client's Opening Handshake, this header field is listed as optional. Other header fields will be handled as described by the aforementioned section. Each WebSocket subprotocol described in Section 5.1.1, Subprotocols, will use a different subprotocol value. In the case of multiple supported subprotocols listed in the client's opening handshake, only one may be selected and will be in the HTTP header sent back in the server's opening handshake.

CERT LACAL-000003 [An LA-CAL that supports the WebSocket Protocol shall set the Sec-WebSocket-Protocol header field to one of the subprotocols defined in the "Subprotocols" table, when sending the server's opening handshake.]

When none of the subprotocols requested in the client's opening handshake can be satisfied, a CAL Server will stop processing the client's opening handshake and return an HTTP response with status 400 Bad Request, as opposed to sending a null subprotocol value. Currently, only one subprotocol is allowed, the OWP subprotocol, but others may be added in future releases of OMS.

CERT LACAL-000004 [An LA-CAL that supports the WebSocket Protocol shall stop processing the client's opening handshake and return an HTTP response with status "400 Bad Request" if the client's opening handshake does not set the Sec-WebSocket-Protocol header field with a subprotocol from the "Subprotocols" table.]

CERT LACAL-000005 [An LA-CAL that supports the WebSocket Protocol shall implement at least one subprotocol in the "Subprotocols" table.]

Table 5.1-1 Subprotocols

| Subprotocol | Sec-WebSocket-Protocol Value |
| --- | --- |
| OMS WebSocket Protocol | owp |

### Subprotocols

#### OMS WebSocket Protocol

This section describes version 1.0 of the OMS WebSocket Protocol (OWP). The version of OWP is composed of major and minor components, separated by a dot, and will only change when substantive changes are made to the subprotocol and does not necessarily change for every release of OMS. The following scheme will be adhered to:

Increase the minor by 1 when any of the following are true:

If an operation is added

If a new key-value pair is added to an existing JSON object

If a new optional field is added to an existing operation such that the semantics and behavior remain the same for existing clients using that operation

Increase the major by 1 and set the minor to 0 when any of the following are true:

If an operation is removed

If the semantics of an operation are altered such that it impacts the expected client behavior

If an existing key-value pair of a JSON object is removed or its semantics are changed

If a new optional or required field is added to an existing operation such that it changes the semantics or behavior of the operation

All other changes, such as clarifying explanations, will not change the version of OWP

CAL Clients that declare OWP as one of their subprotocols during the client's opening handshake will initiate a CAL connection with the CAL Server in conformance with this section if the CAL Server chooses OWP as the subprotocol value.

CAL Messages are sent over this subprotocol in certain operations such as PUB and MSG. These must conform to one of the message formats defined in Section 6, CAL Message Formats. Currently, only one message format is allowed, Section 6.1, OMS JSON, but others may be added in future releases of OMS.

Any references underlined in this section have associated links given by Table 5.1-8, OWP External References.

CERT LACAL-000006 [An LA-CAL that supports the OMS WebSocket Protocol shall enforce that CAL Message Formats for OWP subprotocol operations conform to formats as defined in the "OWP CAL Message Formats" table.]

Table 5.1-2 OWP CAL Message Formats

| CAL Message Formats | Definition |
| --- | --- |
| OMS JSON | See Section 6.1, OMS JSON |

##### Core Concepts

###### Connecting and Initializing

To form an OWP connection, a CAL Client connects to a CAL Server through a new WebSocket connection with exactly one of the values from Table 5.1-1, Subprotocols, for the subprotocol field. After the opening handshake for the connection is completed, the client must send an INIT protocol operation to negotiate the properties of the OWP connection as specified in Section 5.1.1.1.3.1, The INIT Operation, as the first protocol operation. After the INIT protocol operation is sent by a connected CAL Client, an INFO protocol operation is sent by the CAL Server as specified in Section 5.1.1.1.3.5, The INFO Operation. The INFO protocol operation contains information such as the negotiated protocol and schema versions, the server's identifier, and other Service-associated UUIDs.

###### Publish and Subscribe

After the INFO protocol operation is received, a CAL Client uses the PUB and SUB protocol operations to publish and subscribe to CAL Messages. To publish a message, a CAL Client provides a topic and CAL Message as specified in Section 5.1.1.1.3.2, The PUB Operation. To subscribe to a topic and begin receiving messages, a CAL Client provides a unique subscription identifier, the message name, the topic, and an optional subscription group name as specified in Section 5.1.1.1.3.3, The SUB Operation. A CAL Client registers itself as a member of a subscription group by providing the optional subscription group name in the SUB protocol operation. After subscribing to a topic, a CAL Client receives incoming messages with the MSG protocol operation sent by the CAL Server as specified in Section 5.1.1.1.3.6, The MSG Operation. The subscription identifier in the MSG protocol operation refers to a subscription of the connection. For more information on subscription groups and how they function, see Section 4.8, Subscription Groups. To stop receiving messages from a subscription, a CAL Client uses the UNSUB protocol operation with the associated subscription identifier as specified in Section 5.1.1.1.3.4, The UNSUB Operation.

###### Positive and Negative Acknowledgments

After every client protocol operation, the CAL Server sends either a -ERR protocol operation (negative acknowledgment) or a +OK protocol operation (positive acknowledgment) if verbose was true in the INIT protocol operation and the protocol operation completed successfully. The first +OK protocol operation is sent before the INFO protocol operation on verbose connections. If a protocol operation specified does not exist or is not a client protocol operation according to Table 5.1-3, Protocol Operations, then the CAL Server sends a -ERR protocol operation with the error message of Illegal-Operation. If the protocol operation contains the wrong number of fields, then the CAL Server sends a -ERR protocol operation with the error message of Illegal-Argument. Illegal-Argument may also be the error message of a -ERR protocol operation for other reasons, as described in subsequent sections. If the responding server experiences an error internal to the software component or underlying infrastructure while processing any protocol operation, it may either send a -ERR protocol operation with the error message of Internal-Error or fail the connection as defined by RFC 6455 Section 7.1.7, Fail the WebSocket Connection. Additional error messages specific to client protocol operations are described in subsequent sections.

###### Shutdown

The WebSocket connection between a CAL Client and CAL Server may be closed by either peer as dictated by RFC 6455 Section 7, Closing the Connection. WebSocket status codes may be provided during the closing handshake as allowed by RFC 6455 Section 7.4, Status Codes. Status codes used by either client or server are to be documented in the appropriate sections of the Service Contract or Platform Description Document (PDD). After the WebSocket connection is closed, the OWP connection is also considered to be closed and all active subscriptions are disposed of for that connection, as though each subscription was unsubscribed from as specified in Section 5.1.1.1.3.4, The UNSUB Operation.

##### Protocol Conventions

###### Field Delimiters

A protocol operation is a sequence of fields separated by field delimiters with the first field being the operation name. The field delimiters are space (0x20) and horizontal tab (0x09). Consecutive delimiters are to be treated as a single field delimiter.

###### Newlines

Newline characters, such as carriage return (0x0D) and line feed (0x0A), are not considered field delimiters; however, these characters may be used inside of a field.

##### Protocol Operations

Syntax for the protocol operations use angle brackets to denote a required field (e.g. <required field>) and square brackets to denote an optional field (e.g. [optional field]). Only one protocol operation is allowed per WebSocket message.

Table 5.1-3 Protocol Operations

| Name | Sent By | Description |
| --- | --- | --- |
| INIT | Client | Negotiate the properties of the OWP connection |
| PUB | Client | Publish a message on a topic |
| SUB | Client | Subscribe to messages on a topic |
| UNSUB | Client | Unsubscribe from messages on a topic |
| INFO | Server | Inform the client about details of the connection, server, and configuration |
| MSG | Server | Deliver a message on a topic to the client |
| +OK | Server | Acknowledge a well-formed client protocol operation on a verbose connection |
| -ERR | Server | Indicate a protocol operation error |

###### The INIT Operation

A client first sends an INIT operation to negotiate properties of the connection. If a client's requested subprotocol versions, schema version, or Service Identifier cannot be satisfied; INIT is not the first operation sent; or INIT is sent more than once, the server must fail the connection as defined by RFC 6455 Section 7.1.7, Fail the WebSocket Connection. When multiple subprotocol versions are specified, one of those values will be chosen by the server as the subprotocol version. The schema version is the ·value space· of the version attribute in the schema as constrained by CERT SCH-002403.

CERT LACAL-000007 [An LA-CAL that supports the OMS WebSocket Protocol shall implement the OWP subprotocol INIT operation.]

Syntax

INIT <JSON text>

where:

JSON text: A JSON object whose members are defined by Table 5.1-4, INIT Object

Table 5.1-4 INIT Object

| String | Value | Required? | Description |
| --- | --- | --- | --- |
| versions | array of strings | Yes | Desired subprotocol versions |
| schema | string | Yes | Desired schema version for message validation |
| verbose | true or false | No | Whether the server will send +OK after each successful operation; if not provided, the default value is true |
| service_id | string | Yes | Service Identifier associated with this connection |

Errors

In addition to the behaviors described in Section 5.1.1.1.1.3, Positive and Negative Acknowledgments, this operation may yield a -ERR with the error message of any of the following:

Illegal-Argument: If the first field is not a valid JSON text

Illegal-State: If this operation is sent more than once or is not the first operation

Unsupported-Version: If none of the client requested subprotocol versions are supported by the server

Unsupported-Schema: If the client requested schema version is not supported by the server

Unsupported-Service: If the client requested Service Identifier is not supported by the server

###### The PUB Operation

A client publishes messages to the server with the PUB operation. A valid message is delivered to all clients subscribed to the topic, otherwise the publication is dropped.

CERT LACAL-000008 [An LA-CAL that supports the OMS WebSocket Protocol shall implement the OWP subprotocol PUB operation.]

Syntax

PUB <topic> <message>

where:

topic: The topic where the message will be published, matching the regular expression ^[A-Za-z0-9_\-.]+$

message: A CAL Message that conforms to a format in Table 5.1-2, OWP CAL Message Formats

Errors

In addition to the behaviors described in Section 5.1.1.1.1.3, Positive and Negative Acknowledgments, this operation may yield a -ERR with the error message of any of the following:

Illegal-Argument: If topic does not match the given regular expression

Illegal-State: If topic is not an allowed topic name

Invalid-Message: If message does not conform to a format in Table 5.1-2, OWP CAL Message Formats

###### The SUB Operation

A client subscribes to messages on a topic with the SUB operation. Any published message matching those fields will be delivered. When a subscription group is declared, messages are selectively delivered to exactly one group member.

CERT LACAL-000009 [An LA-CAL that supports the OMS WebSocket Protocol shall implement the OWP subprotocol SUB operation.]

Syntax

SUB <subscription id> <message name> <topic> [group]

where:

subscription id: The unique subscription identifier, generated by and local to the CAL Client, matching the regular expression ^[A-Za-z0-9_\-.]+$

message name: The {target namespace} and {name} of a global {element}, specified as "{name}" if {target namespace} is https://www.vdl.afrl.af.mil/programs/oam; otherwise "{{target namespace}}{name}"

topic: The topic that the messages will be received from, matching the regular expression ^[A-Za-z0-9_\-.]+$

group: The optional subscription group name, matching the regular expression ^[A-Za-z0-9_\-.]+$

Errors

In addition to the behaviors described in Section 5.1.1.1.1.3, Positive and Negative Acknowledgments, this operation may yield a -ERR with the error message of any of the following:

Illegal-Argument: If subscription id does not match the given regular expression, if message name is not the {target namespace} and {name} of a global {element}, if topic does not match the given regular expression, or if group does not match the given regular expression

Illegal-State: If subscription id is already in use or if topic is not an allowed topic name

###### The UNSUB Operation

A client unsubscribes to messages associated with a given subscription identifier with the UNSUB operation. A client will no longer receive such messages unless a new subscription is made.

CERT LACAL-000010 [An LA-CAL that supports the OMS WebSocket Protocol shall implement the OWP subprotocol UNSUB operation.]

Syntax

UNSUB <subscription id>

where:

subscription id: The unique subscription identifier, generated by and local to the CAL Client, matching the regular expression ^[A-Za-z0-9_\-.]+$

Errors

In addition to the behaviors described in Section 5.1.1.1.1.3, Positive and Negative Acknowledgments, this operation may yield a -ERR with the error message of any of the following:

Illegal-Argument: If subscription id does not match the given regular expression

Illegal-State: If subscription id is not an active subscription

###### The INFO Operation

The server responds to INIT with an INFO operation. After the properties of the OWP connection are negotiated, the server may send additional INFO operations to signal changes in network topology or associated identifiers.

CERT LACAL-000011 [An LA-CAL that supports the OMS WebSocket Protocol shall implement the OWP subprotocol INFO operation.]

The chosen subprotocol version in the INFO operation is the version specified in Section 5.1.1.1, OMS WebSocket Protocol.

Syntax

INFO <JSON text>

where:

JSON text: A JSON object whose members are defined by Table 5.1-5, INFO Object

Table 5.1-5 INFO Object

| String | Value | Required? | Description |
| --- | --- | --- | --- |
| version | string | Yes | In-use OWP subprotocol version |
| server_id | string | Yes | CAL Server implementation-specific server identifier, matching the regular expression ^[A-Za-z0-9_\-.]+$ |
| uuids | Identifiers Object | Yes | Identifiers associated with the connected Service |
| system_label | string | Yes | System label associated with the initializing Service |
| connect_urls | array of strings | No | Server URLs that the client can connect to |

Table 5.1-6 Identifiers Object

| String | Value | Required? | Description |
| --- | --- | --- | --- |
| system | string | Yes | System UUID that conforms to RFC 4122 Section 3, Namespace Registration Template, associated with the initializing Service |
| service | string | Yes | Service UUID that conforms to RFC 4122 Section 3, Namespace Registration Template, associated with the initializing Service |
| subsystem | string | No | Subsystem UUID that conforms to RFC 4122 Section 3, Namespace Registration Template, associated with the initializing Service |
| capabilities | object whose values are strings | No | Capability names as keys and UUIDs as values that conform to RFC 4122 Section 3, Namespace Registration Template, associated with the initializing Service |
| components | object whose values are strings | No | Component names as keys and UUIDs as values that conform to RFC 4122 Section 3, Namespace Registration Template, associated with the initializing Service |

###### The MSG Operation

A client receives messages associated with a subscription identifier from the MSG operation.

CERT LACAL-000012 [An LA-CAL that supports the OMS WebSocket Protocol shall implement the OWP subprotocol MSG operation.]

Syntax

MSG <subscription id> <message>

where:

subscription id: The unique subscription identifier from the corresponding SUB operation

message: A CAL Message that conforms to a format in Table 5.1-2, OWP CAL Message Formats

###### The +OK Operation

A server positively acknowledges a successful client protocol operation with the +OK operation on a verbose connection.

CERT LACAL-000013 [An LA-CAL that supports the OMS WebSocket Protocol shall implement the OWP subprotocol +OK operation.]

Syntax

+OK

###### The -ERR Operation

A server negatively acknowledges a client protocol operation with the -ERR operation on both verbose and non-verbose connections.

CERT LACAL-000014 [An LA-CAL that supports the OMS WebSocket Protocol shall implement the OWP subprotocol -ERR operation.]

Syntax

-ERR <error message> [details]

where:

error message: Standard error as defined in Table 5.1-7, Errors

details: CAL Server implementation-specific information

Table 5.1-7 Errors

| Error Message | Description |
| --- | --- |
| Unsupported-Version | Occurs when none of the client requested subprotocol versions are supported by the server |
| Unsupported-Schema | Occurs when the client requested schema version is not supported by the server |
| Unsupported-Service | Occurs when the client requested Service Identifier is not supported by the server |
| Illegal-Operation | Occurs when the leading characters of a written WebSocket message do not signify a client protocol operation per Table 5.1-3, Protocol Operations |
| Illegal-Argument | Occurs when the syntax of an existing operation is malformed or otherwise incorrectly specified |
| Illegal-State | Occurs when an operation cannot be fulfilled due to the current state of the connection |
| Internal-Error | Occurs when an operation cannot be fulfilled by the server due to an error internal to the software component or underlying infrastructure |
| Invalid-Message | Occurs when attempting to publish a message that is invalid per the client's specified schema |

##### OWP External References

Table 5.1-8 OWP External References

| Term | Location |
| --- | --- |
| {element} | https://www.w3.org/TR/xmlschema-1/#cElement_Declarations |
| {name} | https://www.w3.org/TR/xmlschema-1/#e-name |
| {target namespace} | https://www.w3.org/TR/xmlschema-1/#e-target_namespace |
| ·value space· | https://www.w3.org/TR/xmlschema-2/#dt-value-space |

# CAL Message Formats

## OMS JSON

JSON is a lightweight, text-based, language-independent data interchange format. This document specifies how JSON text, as prescribed by RFC 8259, is realized from an XML Schema.

This document assumes that all XML Schema documents that are evaluated henceforth comply with the UCI Schema Style and Design Specification OAC-SPC-001. Application of the following OMS JSON format specification on a schema that does not follow the aforementioned specification is undefined.

Any references underlined in this section have associated links given by Table 6.1-4, OMS JSON External References. Values enclosed in double quotes in this section represent the value of a JSON string.

### Global Element Declarations

For a JSON text of a global Element Declaration, all of the following must be true:

It is an object with exactly one member.

The string is given by the appropriate case among the following:

If {target namespace} is https://www.vdl.afrl.af.mil/programs/oam, then "{name}".

Otherwise "{{target namespace}}{name}".

The value is given by the appropriate case among the following:

If {type definition} is a Simple Type Definition, then the text prescribed by Section 6.1.4, Simple Type Definitions.

If {type definition} is a Complex Type Definition, then the text prescribed by Section 6.1.3, Complex Type Definitions.

#### Examples

Representation of an Element Declaration whose {type definition} is a Simple Type Definition

<xs:schema targetNamespace="https://www.vdl.afrl.af.mil/programs/oam">

<xs:element name="MyElement" type="xs:boolean"/>

</xs:schema>

may result in the following JSON text:

{

"MyElement": true

}

Representation of an Element Declaration whose {type definition} is a Complex Type Definition

<xs:schema targetNamespace="https://www.vdl.afrl.af.mil/programs/oam"

xmlns:uci="https://www.vdl.afrl.af.mil/programs/oam">

<xs:element name="MyElement" type="uci:MyComplexType"/>

<xs:complexType name="MyComplexType">

<xs:sequence>

<xs:element name="A" type="xs:string"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

may result in the following JSON text:

{

"MyElement": {

"A": "Hello world"

}

}

Representation of an Element Declaration whose {target namespace} is not https://www.vdl.afrl.af.mil/programs/oam

<xs:schema targetNamespace="uci.program.extension.gov">

<xs:element name="MyExtensionElement" type="xs:string"/>

</xs:schema>

may result in the following JSON text:

{

"{uci.program.extension.gov}MyExtensionElement": "abc"

}

### Particles

For the text of a particle, the appropriate case among the following must be true:

- If {term} is a model group, then the appropriate case among the following:

  - If {compositor} is sequence, then the text prescribed by Section 6.1.2, Particles, for each of the {particles} in any order.

  - If {compositor} is choice, then the text prescribed by Section 6.1.2, Particles, for one of the {particles}.

- If {term} is an Element Declaration, then all of the following must be true:

  - It is a member.

  - The string is given by the appropriate case among the following:

    - If {target namespace} is https://www.vdl.afrl.af.mil/programs/oam, then "{name}".

    - Otherwise "{{target namespace}}{name}".

  - The value is given by the appropriate case among the following:

    - If {max occurs} is greater than one or unbounded, then array whose each value is prescribed per the clause below.

    - Otherwise the appropriate case among the following:

      - If {type definition} is a Simple Type Definition, then the text prescribed by Section 6.1.4, Simple Type Definitions.

      - If {type definition} is a Complex Type Definition, then the appropriate case among the following:

        - If {type definition} can be validly derived as defined in Type Derivation OK (Complex) (§3.4.6), then all of the following:

          - If a type is used in a context where another type was expected, then the member whose string is "$type" and whose value is the appropriate case among the following:

            - If {complexType target namespace} is https://www.vdl.afrl.af.mil/programs/oam, then "{complexType name}."

            - Otherwise "{{complexType target namespace}}{complexType name}."

          - The text prescribed by Section 6.1.3, Complex Type Definitions.

        - Otherwise the text prescribed by Section 6.1.3, Complex Type Definitions.

### Complex Type Definitions

For the text of a Complex Type Definition, all of the following must be true:

- It is an object.

- Its members are prescribed by the appropriate case among the following:

  - If {content type} is empty, then none.

  - If {content type} is a pair consisting of a ·content model· (I.e. a Particle (§2.2.3.2)) and elementOnly, then the text prescribed by Section 6.1.2, Particles, for the ·content model·.

#### Examples

Representation of a Complex Type Definition whose {content type} is empty, e.g. MyEmptyType

<xs:complexType name="MyEmptyType"/>

may result in the following JSON text:

{}

Representation of a Complex Type Definition whose {content type} is the pair of elementOnly and the particle corresponding to the <sequence> among the [children], e.g. MyBasicSequenceType

<xs:complexType name="MyBasicSequenceType">

<xs:sequence>

<xs:element name="A" type="xs:int"/>

<xs:element name="B" type="xs:string" minOccurs="0" maxOccurs="2">

</xs:sequence>

</xs:complexType>

may result in any of the following JSON texts:

{

"A": 123,

"B": [ "first", "second" ]

}

{

"B": [ "first" ],

"A": 123

}

{

"B": [],

"A": 123

}

{

"A": 123

}

Representation of a Complex Type Definition whose {content type} is the pair of elementOnly and a particle whose {term} is a model group whose {compositor} is sequence and whose {particles} are the particle of the {content type} of the type definition ·resolved· to by the ·actual value· of the base [attribute] followed by the ·effective content·, e.g. MyDerivedType

<xs:complexType name="MyBaseType">

<xs:sequence>

<xs:element name="A" type="xs:double"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MyDerivedType">

<xs:complexContent>

<xs:extension base="MyBaseType">

<xs:sequence>

<xs:element name="B" type="xs:string"/>

<xs:element name="C" type="xs:hexBinary"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

may result in the following JSON text:

{

"A": 3.14,

"B": "PI",

"C": "ee"

}

or MyCombinatorType

<xs:complexType name="MyAbstractBaseType" abstract="true">

<xs:sequence>

<xs:element name="A" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MyConcreteType">

<xs:complexContent>

<xs:extension base="MyAbstractBaseType">

<xs:sequence>

<xs:element name="B" type="xs:int" maxOccurs="4">

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="MyOtherType">

<xs:complexContent>

<xs:extension base="MyAbstractBaseType"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="MyCombinatorType">

<xs:sequence>

<xs:element name="C" type="MyAbstractBaseType" maxOccurs="2"/>

</xs:sequence>

</xs:complexType>

may result in the following JSON text:

{

"C": [

{

"A": "From the base type",

"$type": "MyConcreteType",

"B": [ 1, 2, 3, 4 ]

},

{

"$type": "MyOtherType",

"A": "Not completely empty"

}

]

}

Representation of a Complex Type Definition whose ·effective content· is empty and the {content type} is the type definition ·resolved· to by the ·actual value· of the base [attribute], e.g. MyImplType

<xs:complexType name="MyBaseType">

<xs:sequence>

<xs:element name="MyField1" type="string"/>

<xs:element name="MyField2" type="string" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MyImplType">

<xs:complexContent>

<xs:extension base="MyBaseType"/>

</xs:complexContent>

</xs:complexType>

may result in any of the following JSON texts:

{

"MyField1": "My Field 1"

}

{

"MyField1": "My Field 1",

"MyField2": "My Field 2",

"$type": "MyImplType"

}

Representation of a Complex Type Definition whose {content type} is the pair of elementOnly and a particle whose {term} is a model group whose {compositor} is choice, e.g. MyChoiceType

<xs:complexType name="MyFirstType">

<xs:sequence>

<xs:element name="C" type="xs:unsignedInt"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MySecondType">

<xs:sequence>

<xs:element name="D" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MyChoiceType">

<xs:choice>

<xs:element name="A" type="MyFirstType"/>

<xs:element name="B" type="MySecondType" maxOccurs="3"/>

</xs:choice>

</xs:complexType>

may result in any of the following JSON texts:

{

"A": {

"C": 134

}

}

{

"B": [

{

"D": "abc"

},

{

"D": "123"

}

]

}

### Simple Type Definitions

A Simple Type Definition is mapped based only on its {primitive type definition}.

For the text of the ·actual value· of a Simple Type Definition, the appropriate case among the following must be true:

- If {primitive type definition} is xs:boolean, then true or false depending on the ·actual value·.

- If {primitive type definition} is xs:float or xs:double, then number or one of the following strings: "NaN", "Infinity", or "-Infinity".

- If {primitive type definition} is xs:decimal, then number.

- If {simpleType name} is UniversallyUniqueIdentifierType and {simpleType target namespace} is https://www.vdl.afrl.af.mil/programs/oam, then string whose value conforms to RFC 4122 Section 3, Namespace Registration Template.

- Otherwise string.

#### Examples

Representation of a Simple Type Definition whose {primitive type definition} is xs:string

<xs:simpleType name="MyStringType">

<xs:restriction base="xs:normalizedString"/>

</xs:simpleType>

may result in the following JSON text:

"abcdefghijkl"

Representation of a Simple Type Definition whose {primitive type definition} is xs:boolean

<xs:simpleType name="MyBooleanType">

<xs:restriction base="xs:boolean"/>

</xs:simpleType>

may result in either the following JSON texts:

true

or

false

Representation of a Simple Type Definition whose {primitive type definition} is xs:decimal

<xs:simpleType name="MyDecimalType">

<xs:restriction base="xs:int"/>

</xs:simpleType>

may result in either of the following JSON texts:

9999

-9999

Representation of a Simple Type Definition whose {primitive type definition} is xs:float or xs:double

<xs:simpleType name="MyFloatType">

<xs:restriction base="xs:float"/>

</xs:simpleType>

may result in any the following JSON texts:

99.99

9.9E99

9999

-9999e-99

"NaN"

"Infinity"

"-Infinity"

Representation of a Simple Type Definition whose {simpleType name} is UniversallyUniqueIdentifierType and {simpleType target namespace} is https://www.vdl.afrl.af.mil/programs/oam

<xs:schema targetNamespace="https://www.vdl.afrl.af.mil/programs/oam">

<xs:simpleType name="UniversallyUniqueIdentifierType"/>

</xs:schema>

may result in any of the following JSON texts:

"697eba54-6f37-11ed-a1eb-0242ac120002"

"539a5e3a-1c72-4c19-83c7-0024c0a15d8c"

"00000000-0000-0000-0000-000000000000"

### Validation

An OMS JSON message is valid if and only if its schema-validity assessment is valid with respect to its XML Schema. This determination relies on an associated document information item for the message as prescribed by Section 6.1.5.1, Document Information Item, and its descendant element information items.

#### Document Information Item

Table 6.1-1 Document Information Item

| Property Name | Property Value |
| --- | --- |
| [children] | A list of exactly one element information item, the [document element]. |
| [document element] | The element information item for the single member prescribed by Section 6.1.5.3, Element Information Items of a Member. |
| [notations] | empty set |
| [unparsed entities] | empty set |
| [base URI] | absent |
| [character encoding scheme] | UTF-8 |
| [standalone] | No value |
| [version] | "1.0" |
| [all declarations processed] | false |

#### Namespace Name and Local Name of a Member

The namespace name of a member is given by the appropriate case among the following:

If the string starts with "{" (0x7B), then the characters between braces "{" and "}" (0x7D).

Otherwise, https://www.vdl.afrl.af.mil/programs/oam.

The local name of a member is given by the appropriate case among the following:

If the string starts with "{", then all the characters after the closing brace "}".

Otherwise, the entire string.

#### Element Information Items of a Member

The element information items of a member are prescribed by Table 6.1-2, Element Information Items of a Member.

Table 6.1-2 Element Information Items of a Member

| Property Name | Property Value |
| --- | --- |
| [namespace name] | The namespace name as defined by Section 6.1.5.2, Namespace Name and Local Name of a Member. |
| [local name] | The local name as defined by Section 6.1.5.2, Namespace Name and Local Name of a Member. |
| [prefix] | No value |
| [children] | A list given by the appropriate case among the following: 1. If value is an object, then the element information items given by mapping each member according to Section 6.1.5.3, Element Information Items of a Member, in an order ·valid· with respect to its associated {content type}'s particle, as defined in Element Sequence Locally Valid (Particle) (§3.9.4). 2. If value is an array, then, for each value in the array, the element information item prescribed by Section 6.1.5.3, Element Information Items of a Member, except [namespace name] is the [parent]'s [namespace name] and [local name] is the [parent]'s [local name]. 3. Otherwise, the character information items prescribed by Section 6.1.5.4, Character Information Items of Values. |
| [attributes] | empty set |
| [namespace attributes] | empty set |
| [in-scope namespaces] | empty set |
| [base URI] | No value |
| [parent] | The document information item or element information items which contains this information item in its [children] property. |

#### Character Information Items of Values

The character information items of a value is given by the appropriate case among the following:

If value is string, then the list of character information items from each char production of the string (i.e., without leading and trailing quotation marks) as prescribed by Table 6.1-3, Character Information Items of Values.

If value is number, true, or false, then the list of character information items from each character of the value as prescribed by Table 6.1-3, Character Information Items of Values.

Otherwise, none.

Table 6.1-3 Character Information Items of Values

| Property Name | Property Value |
| --- | --- |
| [character code] | The ISO 10646 character code of the character. |
| [element content whitespace] | true |
| [parent] | The element information item which contains this information item in its [children] property. |

### OMS JSON External References

Table 6.1-4 OMS JSON External References

| Term | Location |
| --- | --- |
| actual value | https://www.w3.org/TR/xmlschema-1/#key-vv |
| array | https://datatracker.ietf.org/doc/html/rfc8259#section-5 |
| [attribute] | https://www.w3.org/TR/xml-infoset/#infoitem.element |
| character information items | https://www.w3.org/TR/xml-infoset/#infoitem.character |
| [children] | https://www.w3.org/TR/xml-infoset/#infoitem.element |
| Complex Type Definition | https://www.w3.org/TR/xmlschema-1/#Complex_Type_Definition_details |
| {complexType name} | https://www.w3.org/TR/xmlschema-1/#ct-name |
| {complexType target namespace} | https://www.w3.org/TR/xmlschema-1/#ct-target_namespace |
| {compositor} | https://www.w3.org/TR/xmlschema-1/#compositor |
| ·content model· | https://www.w3.org/TR/xmlschema-1/#compositor |
| {content type} | https://www.w3.org/TR/xmlschema-1/#content_type |
| document information item | https://www.w3.org/TR/xml-infoset/#infoitem.document |
| ·effective content· | https://www.w3.org/TR/xmlschema-1/#key-exg |
| Element Declaration | https://www.w3.org/TR/xmlschema-1/#Element_Declaration_details |
| element information item | https://www.w3.org/TR/xml-infoset/#infoitem.element |
| element information items | https://www.w3.org/TR/xml-infoset/#infoitem.element |
| Element Sequence Locally Valid (Particle) (§3.9.4) | https://www.w3.org/TR/xmlschema-1/#cvc-particle |
| false | https://datatracker.ietf.org/doc/html/rfc8259#section-3 |
| local name | https://www.w3.org/TR/xmlschema-1/#q-local |
| {max occurs} | https://www.w3.org/TR/xmlschema-1/#p-max_occurs |
| member | https://datatracker.ietf.org/doc/html/rfc8259#section-4 |
| model group | https://www.w3.org/TR/xmlschema-1/#Model_Group_details |
| {name} | https://www.w3.org/TR/xmlschema-1/#e-name |
| namespace name | https://www.w3.org/TR/xml-names/#dt-NSName |
| number | https://datatracker.ietf.org/doc/html/rfc8259#section-6 |
| object | https://datatracker.ietf.org/doc/html/rfc8259#section-4 |
| Particle (§2.2.3.2) | https://www.w3.org/TR/xmlschema-1/#Particle |
| particle | https://www.w3.org/TR/xmlschema-1/#Particle_details |
| {particles} | https://www.w3.org/TR/xmlschema-1/#particles |
| {primitive type definition} | https://www.w3.org/TR/xmlschema-1/#st-primitive_type_definition |
| ·resolved· | https://www.w3.org/TR/xmlschema-1/#src-resolve |
| schema-validity assessment | https://www.w3.org/TR/xmlschema-1/#validation_outcome |
| <sequence> | https://www.w3.org/TR/xmlschema-1/#element-sequence |
| Simple Type Definition | https://www.w3.org/TR/xmlschema-1/#Simple_Type_Definition_details |
| {simpleType name} | https://www.w3.org/TR/xmlschema11-1/#std-name |
| {simpleType target namespace} | https://www.w3.org/TR/xmlschema11-1/#std-target_namespace |
| string | https://datatracker.ietf.org/doc/html/rfc8259#section-7 |
| {target namespace} | https://www.w3.org/TR/xmlschema-1/#e-target_namespace |
| {term} | https://www.w3.org/TR/xmlschema-1/#term |
| true | https://datatracker.ietf.org/doc/html/rfc8259#section-3 |
| {type definition} | https://www.w3.org/TR/xmlschema-1/#type_definition |
| Type Derivation OK (Complex) (§3.4.6) | https://www.w3.org/TR/xmlschema-1/#cos-ct-derived-ok |
| value | https://datatracker.ietf.org/doc/html/rfc8259#section-3 |
| ·valid· | https://www.w3.org/TR/xmlschema-1/#key-vn |
| XML Schema | https://www.w3.org/TR/xmlschema-1/ |
| xs:boolean | https://www.w3.org/TR/xmlschema-2/#boolean |
| xs:decimal | https://www.w3.org/TR/xmlschema-2/#decimal |
| xs:double | https://www.w3.org/TR/xmlschema-2/#double |
| xs:float | https://www.w3.org/TR/xmlschema-2/#float |
| xs:string | https://www.w3.org/TR/xmlschema-2/#string |

# Acronyms and Symbols

For the list of OMS acronyms, refer to the document OMSC-STD-002, Abbreviations and Glossary.

# Glossary

For the glossary of terms, refer to the document OMSC-STD-002, Abbreviations and Glossary.